# Chapter1

## Module 1: IT Specialist Skills & Troubleshooting (Lesson 1A)

### 1. Role of an IT Specialist

- Definition: A professional who installs, maintains, troubleshoots, and supports IT systems, networks, and devices.

- Responsibilities:

  - Assist users with hardware, software, and connectivity issues.

  - Maintain, upgrade, and troubleshoot desktops, laptops, mobile devices, and servers.

  - Ensure network and system functionality and security.

  - Document issues, solutions, and procedures.

  - Communicate technical information clearly to non-technical users.

- Example Scenarios:

  - Connecting a laptop to an email account.

  - Troubleshooting database server connectivity.

  - Diagnosing perceived internet outages.

### 2. Skills Required for IT Specialists

- Technical Skills:

  - Knowledge of hardware, software, and operating systems.

  - Networking fundamentals (TCP/IP, LAN/WAN, Wi-Fi).

  - Security basics (authentication, encryption, backups).

- Problem-Solving Skills:

  - Diagnose issues logically.

  - Apply troubleshooting methodology systematically.

  - Use research and testing to confirm root causes.

- Communication Skills:

  - Explain technical issues to non-technical users.

  - Document findings clearly.

  - Collaborate with colleagues and vendors.

- Professionalism:

  - Customer service and empathy.

  - Time management and organization.

  - Ethical behavior and confidentiality.

### 3. CompTIA Troubleshooting Methodology

- Identify the Problem

  - Gather information from the user:

    - Error messages.

    - Duration and frequency of the problem.

    - Recent changes to hardware/software.

    - Environmental or infrastructure changes.

    - Steps already attempted by the user.

  - Perform backups before making changes.

  - Begin documentation of the problem.

  - Prioritize cases based on impact.

- Establish a Theory of Probable Cause

  - Consider the obvious first: power, cables, simple misconfigurations.

  - Use prior knowledge, system behavior, and symptoms to hypothesize.

  - Conduct research if needed (internal documentation, vendor resources, forums, web search).

  - Physically inspect hardware: check for visible damage, listen for abnormal sounds.

  - Attempt to reproduce the problem to understand its cause.

- Test the Theory to Determine Cause

  - Confirm the hypothesis with testing.

  - If theory is correct → proceed to solution.

  - If theory is incorrect → form a new theory or escalate.

- Establish a Plan of Action and Implement Solution

  - Consider:

    - Resources required.

    - Cost and time constraints.

    - Potential impacts on other systems.

  - Generic approaches:

    - Repair: Fix the failing component.

    - Replace: Substitute hardware/software.

    - Workaround: Temporary solution if repair/replacement is impractical.

  - Obtain proper authorization before implementing changes.

  - Follow vendor instructions if required.

- Verify Full-System Functionality

  - Test the system to ensure:

    - Problem is fully resolved.

    - No new issues have been introduced.

  - Examples:

    - Use the component/activity that caused the original issue.

    - Check logs, indicators, or diagnostic tools.

    - Reinstall or update drivers/software if needed.

  - Get user confirmation before closing the ticket.

- Document Findings, Actions, Lessons Learned, and Outcomes

  - Record:

    - What the problem was.

    - Steps taken to resolve it.

    - Preventive measures implemented.

    - Lessons learned for future reference.

  - Maintain clarity, grammar, and professionalism.

  - Use logs to:

    - Build knowledge bases / FAQs.

    - Provide evidence of IT support work.

    - Track recurring problems and system weaknesses.

### 4. Preventive Measures

- Implement actions to avoid recurrence:

  - Replace faulty hardware and check environmental factors.

  - Apply antivirus and security updates.

  - Educate users on best practices and malware prevention.

- Preventive maintenance ensures system reliability and reduces downtime.

### 5. Key Tips

- Question the obvious: Many problems have simple causes.

- Break problems into categories: Power, hardware, software, network, user actions.

- Use “known good” substitutes to quickly test hardware or connections.

- Escalate when necessary:

  - To senior staff, SMEs, vendors, or external contractors.

  - Always document attempts before escalation.

# Cable Types

## USB

Universal Serial Bus

Longer cables have higher chances of resistance and speed and signal deterioration

Type A – Most commonly used on desktops and laptops

Type C – Modern connector

Type B 2.0 and 3.0 are NOT interchangeable

### USB 2.0

Name | Standard | Speed | Max Cable Length | Power

LowSpeed | USB 1.0 | 1.5 Mbps | 5M / 15Ft | 0.5amps

FullSpeed | USB 1.1 | 12 Mbps | 5M / 15Ft | 0.5amps

HiSpeed | USB 2.0 | 480 Mbps | 5M / 15Ft | 0.5amps

### USB 3.0

Name | Standard | Speed | Max Cable Length | Power

Super Speed USB | USB 3.2 Gen 1 | 5Gbps | 3M / 9Ft | 4.5W

Super Speed USB | USB 3.2 Gen 2x1  | 10Gbps | 3M / 9Ft | 4.5W

Super Speed USB | USB 3.2 Gen 2x2 | 20Gbps | 3M / 9Ft | 4.5W

### USB 4

40Gbps | 3M / 9Ft | 7.5W

## Video Cables

### HDMI

High-Definition Multimedia Interface

- 1080p up to 4K and 8K with 60, 120 or 144 Hertz

- HDMI Digital Content Protection (HDCP) – Allows a device to validate the connection

  - Type A – Full-size Connector

  - Type C – Mini Connector

  - Type D – Micro Connector

#### Category 1

- Standard – Used for video content – 1080P @ 60 Hertz

#### Category 2

- High Speed – Has great length of distance and high resolution

  - Premium High Speed up to 18Gbps

  - Ultra-High Speed up to 48 Gbps

### DisplayPort

Has the same capabilities as HDMI

- Full Size DisplayPort – FDP

- Mini DisplayPort – MDP

- Can support data transfer up to 20 Gbps

### DVI

Digital Visual Interface – Supports analog and digital outputs

DVI A – Analog only

DVI D – Digital only

DVI I – Both analog and digital

### VGA

Video Graphics Array – Uses a 15-pin standard analog video interface port

### Thunderbolt

Acts like a display interface and can data transfer

#### Thunderbolt 1 & 2

- Use a physical connector that’s backwards compatible with MDP

#### Thunderbolt 3

- Changed the physical interface to use the same port connector as USB-C

- USB-C does not always support Thunderbolt 3, but all Thunderbolt 3 supports USB-C

- Thunderbolt 3 – 1.6 Ft up to 40 Gbps

#### Lightning

- Specific proprietary created by Apple for mobile devices

## Storage Cables

### SATA

Serial Advanced Technology Attachment – Standard method of connecting a storage device to a motherboard

SATA 7-Pin Data Cable – Does not supply power

SATA 15-Pin Power Connector – Provides power

SATA 1 – up to 1Gbps | 150 MBps

SATA 2 – up to 3Gbps | 300 MBps

SATA 3 – up to 6Gbps | 600 MBps

Limitation will usually be the Hard Drive, not the cable

### eSATA

external Serial Advanced Technology Attachment – Cable outside of the case

eSATA2 – 3Gbps

eSATA3 – 6Gbps

Parallel Advanced Technology Attachment (PATA) – Uses IDE cables and connectors and standards but is differentiated from SATA – 40-Pin ribbon cable/MOLEX Power Connector 4-Pin

## Motherboards

Motherboard – Printed circuit board that contains computer components and provides connectors

Input – Process of accepting data in a form that the computer can use

Output – Process of displaying the processed data or information

Processing – Actions performed by the CPU when receiving information

Storage – Process of saving or retaining digital data, temporarily or permanently

## Installing the Motherboard and CPU

- Review the motherboard’s documentation

- Position the motherboard to align with rear of the case

- Insert the standoffs to match the hole locations for the motherboard

- Install the processor and memory modules before installing the motherboard

- Verify the standoffs are properly aligned prior to installing the motherboard

- Secure the standoffs using the appropriate screw type

- Install the power supply, disk drives, add-on cards and other components

## Form Factors

ATX – Advanced Technology eXtended – Full-size motherboard and measures 12x9.6 in size

Mini ATX – Smaller than the ATX but contains the same features. 11.2x8.2

Micro ATX – 9.6 squared – Only has 4 expansion slots instead of up to 7

Mini ITX – Information Technology eXtended 6.7 squared only 1 expansion slot

## Expansion Cards

PCI 32-bit – Support a max bus speed of 33MHZ or 133MBps

PCI-X – 64-bit expansion card 133MHz

PCI-X 2.0 266MHz up to 533 MHz

PCIe (Peripheral Component Interconnect Express) – replaces PCI, PCI-X and APG – Connects to the bus to get data to and from the motherboard for external devices

PCIe x1. PCIe x4, PCIe x8. PCIe x16

X1 – used for modems, network cards, wireless cards, input/output devices and audio cards

X16 – graphics cards

All PCIe slots provide 25 watts of power

PCIe x16 provides 75 watts of power

Up plugging – Putting a smaller card in a larger slot

Down plugging – Putting larger card in a smaller slot

Mini PCIe – Standard PCIe card with smaller form factor – used inside of laptops, specifically for wireless networking