## 1.1 Laptop Hardware CompiTIA A+ Professor Messer

## 1. Laptop Design & Troubleshooting

- Laptops pack many components into a small form factor.

- Repairs and troubleshooting require precision because parts are small and tightly packed.

- Ease of repair varies by manufacturer:

  - Some laptops = easy access to RAM, storage, battery.

  - Others = sealed, hard to open, parts soldered.

## 2. Laptop Batteries

### Types

- Lithium-Ion (Li-Ion)

- Lithium-Ion Polymer (Li-Po)
Both have no memory effect → can be charged anytime.

### Form Factor

- Battery shapes and connectors vary by model.

- Must buy the exact battery model for the laptop.

### Modular vs. Integrated Batteries

- Modular (removable):

  - User removable.

  - Pops out with clips/buttons.

- Integrated (non-removable):

  - Built inside the chassis.

  - Requires technician disassembly to replace.

## 3. Laptop Keyboards

### Common Issues

- Keyboards are heavily used → often replaced.

### Replacement

- Usually easy: remove bezel/screws → lift keyboard.

- Connected with a ribbon cable to the system board.

### Troubleshooting

- Use an external USB keyboard to:

  - Determine OS vs hardware issue.

  - Continue working if built-in keyboard is faulty.

### Key Characteristics

- Compact layout.

- Function keys for secondary features.

- Some have numeric keypads (on wider laptops).

### Keycap Repairs

- Laptop keycaps are delicate.

- Follow manufacturer instructions to avoid breaking hinge mechanisms.

## 4. Laptop Memory (RAM)

### Types

- Laptops use SO-DIMM (Small Outline DIMM).

### Modular vs. Soldered RAM

- Modular SO-DIMM:

  - Can be removed/replaced.

  - Located behind an access panel.

- Soldered RAM:

  - Permanently attached to motherboard.

  - Cannot upgrade without replacing the entire board.

### Installation

- Insert RAM at an angle → push down → clips hold it in place.

## 5. Laptop Storage

### Types

- HDDs (Hard Disk Drives)

  - Older laptops use 2.5 inch drives.

- SSDs (Solid State Drives)

  - No moving parts, faster & more reliable.

  - Two main types:

    - 2.5 inch SSD (older)

    - M.2 SSD (newer, much smaller)

### Connections

- 2.5 inch drives require:

  - SATA data cable

  - SATA power cable

- M.2 drives use a single slot connector + 1 screw.

### Installation

- Remove back panel.

- For 2.5 inch:

  - Unscrew → slide out the drive.

- For M.2:

  - Insert card → push down → secure with screw.

## 6. Cloning & Migrating Storage

- Replacing HDD with SSD is a common upgrade.

- Methods:

  - Fresh OS install + move user files.

  - Drive imaging / cloning software (faster).

- Some SSDs include free cloning tools.

- Can clone:

  - Directly from old to new drive, or

  - Create an image and restore it.

## 7. Laptop Wireless Communication

### Wi-Fi (802.11)

- Many laptops use integrated Wi-Fi chips.

- Older systems used mini PCI or mini PCIe cards.

### Wireless Card Installation

- Insert card like RAM.

- Connect antenna wires (usually black = main, white = aux).

- Antennas run inside the laptop display bezel.

### Bluetooth

- Used for short-range PAN (Personal Area Network).

- Often part of the Wi-Fi card or built into the motherboard.

## 8. Biometrics

### Authentication Methods

- Fingerprint recognition.

- Face recognition (Windows Hello Face).

### Requirements

- OS support (e.g., Windows Hello).

- Hardware support:

  - Fingerprint reader

  - Webcam with IR sensor (for advanced facial ID)

### Benefits

- More secure and convenient than passwords.

## 9. NFC (Near Field Communication)

- Short-range communication (< 4 cm).

- Uses:

  - Contactless payments.

  - Badge authentication (hospitals, warehouses).

  - Quick device pairing.

## 10. Laptop Cameras & Microphones

- Built-in camera usually at top center of display.

- Laptops include microphones next to the camera.

- Can use external cameras for higher quality.

# ✅ Exam-Ready Summary (Quick Review)

- SO-DIMM = laptop RAM form factor.

- M.2 = small SSD interface used in modern laptops.

- Integrated battery requires full disassembly to replace.

- Cloning helps migrate data from HDD to SSD quickly.

- Antenna wires for Wi-Fi/Bluetooth run through display bezel.

- Biometrics (fingerprint/face) require OS + hardware support.

- NFC = very short-range wireless for authentication and payments.

# 📘 Chapter 1.1 — Laptop Hardware: Complete Exam-Style Questions & Answers

## 1️⃣ Multiple Choice Questions (20)

- Which type of RAM is commonly used in laptops?
A. DIMM
B. SODIMM
C. MicroDIMM
D. ECC DIMM
Answer: B

- Which storage type is fastest for a laptop?
A. HDD
B. SATA SSD
C. M.2 NVMe SSD
D. SD Card
Answer: C

- Laptop batteries most commonly use which chemistry?
A. NiMH
B. NiCd
C. Li-Ion
D. Lead-acid
Answer: C

- What connects the laptop keyboard to the motherboard?
A. SATA cable
B. Ribbon cable
C. USB cable
D. HDMI cable
Answer: B

- Which interface is used by modern M.2 SSDs?
A. SATA
B. PCIe NVMe
C. IDE
D. USB
Answer: B

- Laptop hinges serve which purpose?
A. Thermal dissipation
B. Opening and closing the screen
C. Power regulation
D. RAM upgrade access
Answer: B

- Laptop touchpads commonly use:
A. Mechanical switches
B. Capacitive sensing
C. Optical sensors
D. Pressure plates
Answer: B

- The embedded controller (EC) in a laptop:
A. Controls GPU
B. Manages keyboard, power, touchpad
C. Reads/writes hard drive
D. Outputs display signals
Answer: B

- TN displays are known for:
A. Best color accuracy
B. Fast response time, poor viewing angles
C. High resolution only
D. Touch functionality
Answer: B

- Which component is NOT typically user-upgradeable?
A. RAM
B. SSD
C. CPU
D. Wi-Fi card
Answer: C

- Which USB standard is most common in modern laptops?
A. USB 2.0
B. USB 3.x / USB-C
C. PS/2
D. Serial
Answer: B

- What is the function of thermal paste in laptops?
A. Protects RAM
B. Improves heat transfer between CPU/GPU and heatsink
C. Insulates battery
D. Reduces fan noise
Answer: B

- A docking station is used to:
A. Upgrade CPU
B. Add external ports and monitors
C. Replace RAM
D. Format drives
Answer: B

- Laptop batteries that can be removed by the user are called:
A. Modular
B. Integrated
C. Soldered
D. Non-removable
Answer: A

- What is the common voltage output for laptop chargers?
A. 5V
B. 9V
C. 19V
D. 48V
Answer: C

- A laptop Wi-Fi card connects typically via:
A. Mini PCIe
B. SATA
C. M.2
D. USB
Answer: A or C

- What is the main difference between a 2.5-inch SSD and an M.2 SSD?
A. Price
B. Form factor and interface
C. Battery type
D. Keyboard type
Answer: B

- Which laptop component often fails first due to heavy usage?
A. RAM
B. Keyboard
C. CPU
D. SSD
Answer: B

- The inverter in older laptops:
A. Converts AC to DC
B. Converts DC to AC for CCFL backlight
C. Powers USB devices
D. Boosts battery voltage
Answer: B

- Which GPU is found in laptops without a dedicated graphics card?
A. External GPU
B. Integrated GPU
C. PCIe GPU
D. Discrete GPU
Answer: B

## 2️⃣ Performance-Based / Lab Simulation Questions (5)

- Task: Simulate removing a SODIMM RAM module and replacing it with a new one.
Answer Steps:

- Power off laptop and remove battery (if removable)

- Open RAM compartment

- Release clips and remove old RAM

- Insert new RAM until clips lock

- Task: Replace a 2.5-inch HDD with an SSD in a laptop.
Answer Steps:

- Power off and remove battery

- Open back panel

- Remove screws holding HDD

- Install SSD in same bracket, connect SATA and power

- Close panel and power on

- Task: Install an M.2 NVMe SSD in a laptop.
Answer Steps:

- Remove back cover

- Locate M.2 slot

- Insert SSD at 30° angle

- Push down and screw it in

- Task: Disconnect and reconnect a laptop keyboard ribbon cable.
Answer Steps:

- Open laptop bezel

- Unlock ribbon connector

- Gently pull out ribbon

- Insert new keyboard ribbon and lock connector

- Task: Replace a modular laptop battery.
Answer Steps:

- Slide battery release latch

- Remove battery

- Insert new battery until it clicks

## 3️⃣ Scenario-Based Questions (10)

- A laptop keyboard is unresponsive but an external USB keyboard works. What should you check?
Answer: Check the internal keyboard ribbon connection or replace keyboard

- A laptop battery drains quickly even after replacement. What should you check?
Answer: Power-hungry apps, battery calibration, or faulty power management

- Laptop display flickers intermittently. Possible causes?
Answer: Loose display cable, inverter issue (for older displays), or faulty screen

- Laptop running slow due to HDD. Recommended upgrade?
Answer: Replace HDD with SSD

- Wi-Fi card replaced but connection is weak. Likely problem?
Answer: Antenna wires not connected properly

- One key on the keyboard sticks. Best solution?
Answer: Replace key carefully or entire keyboard

- Laptop overheats frequently. First troubleshooting steps?
Answer: Clean fan, vents, and check thermal paste

- New M.2 SSD installed, but laptop doesn’t boot. Why?
Answer: OS not installed or BIOS not detecting drive

- User reports battery not charging. Battery is integrated. What to do?
Answer: Technician disassembles laptop to replace battery

- Touchpad not responding. First action?
Answer: Check device drivers or settings

## 4️⃣ Matching Questions (5)

Match Laptop Component → Function

| Component | Function |
| --- | --- |
| A. SODIMM RAM | 1. Storage |
| B. SSD | 2. Temporary memory |
| C. Battery | 3. Provides power |
| D. Wi-Fi Card | 4. Wireless connectivity |
| E. Embedded Ctrl | 5. Controls keyboard/power functions |

Answer:
A → 2
B → 1
C → 3
D → 4
E → 5

## 5️⃣ True / False Questions (7)

- Laptop RAM is always soldered to the motherboard. False

- M.2 SSDs are smaller than 2.5-inch SSDs. True

- Integrated GPUs are part of the CPU. True

- Scissor-switch keyboards are common in laptops. True

- Thermal paste is applied between CPU/GPU and RAM. False

- Modular batteries are easier to replace than integrated batteries. True

- Laptop hinges are purely decorative. False