## 1. Laptop Design & Troubleshooting

- Laptops pack many components into a small form factor.

- Repairs and troubleshooting require precision because parts are small and tightly packed.

- Ease of repair varies by manufacturer:

  - Some laptops = easy access to RAM, storage, battery.

  - Others = sealed, hard to open, parts soldered.

## 2. Laptop Batteries

### Types

- Lithium-Ion (Li-Ion)

- Lithium-Ion Polymer (Li-Po)
Both have no memory effect → can be charged anytime.

### Form Factor

- Battery shapes and connectors vary by model.

- Must buy the exact battery model for the laptop.

### Modular vs. Integrated Batteries

- Modular (removable):

  - User removable.

  - Pops out with clips/buttons.

- Integrated (non-removable):

  - Built inside the chassis.

  - Requires technician disassembly to replace.

## 3. Laptop Keyboards

### Common Issues

- Keyboards are heavily used → often replaced.

### Replacement

- Usually easy: remove bezel/screws → lift keyboard.

- Connected with a ribbon cable to the system board.

### Troubleshooting

- Use an external USB keyboard to:

  - Determine OS vs hardware issue.

  - Continue working if built-in keyboard is faulty.

### Key Characteristics

- Compact layout.

- Function keys for secondary features.

- Some have numeric keypads (on wider laptops).

### Keycap Repairs

- Laptop keycaps are delicate.

- Follow manufacturer instructions to avoid breaking hinge mechanisms.

## 4. Laptop Memory (RAM)

### Types

- Laptops use SO-DIMM (Small Outline DIMM).

### Modular vs. Soldered RAM

- Modular SO-DIMM:

  - Can be removed/replaced.

  - Located behind an access panel.

- Soldered RAM:

  - Permanently attached to motherboard.

  - Cannot upgrade without replacing the entire board.

### Installation

- Insert RAM at an angle → push down → clips hold it in place.

## 5. Laptop Storage

### Types

- HDDs (Hard Disk Drives)

  - Older laptops use 2.5 inch drives.

- SSDs (Solid State Drives)

  - No moving parts, faster & more reliable.

  - Two main types:

    - 2.5 inch SSD (older)

    - M.2 SSD (newer, much smaller)

### Connections

- 2.5 inch drives require:

  - SATA data cable

  - SATA power cable

- M.2 drives use a single slot connector + 1 screw.

### Installation

- Remove back panel.

- For 2.5 inch:

  - Unscrew → slide out the drive.

- For M.2:

  - Insert card → push down → secure with screw.

## 6. Cloning & Migrating Storage

- Replacing HDD with SSD is a common upgrade.

- Methods:

  - Fresh OS install + move user files.

  - Drive imaging / cloning software (faster).

- Some SSDs include free cloning tools.

- Can clone:

  - Directly from old to new drive, or

  - Create an image and restore it.

## 7. Laptop Wireless Communication

### Wi-Fi (802.11)

- Many laptops use integrated Wi-Fi chips.

- Older systems used mini PCI or mini PCIe cards.

### Wireless Card Installation

- Insert card like RAM.

- Connect antenna wires (usually black = main, white = aux).

- Antennas run inside the laptop display bezel.

### Bluetooth

- Used for short-range PAN (Personal Area Network).

- Often part of the Wi-Fi card or built into the motherboard.

## 8. Biometrics

### Authentication Methods

- Fingerprint recognition.

- Face recognition (Windows Hello Face).

### Requirements

- OS support (e.g., Windows Hello).

- Hardware support:

  - Fingerprint reader

  - Webcam with IR sensor (for advanced facial ID)

### Benefits

- More secure and convenient than passwords.

## 9. NFC (Near Field Communication)

- Short-range communication (< 4 cm).

- Uses:

  - Contactless payments.

  - Badge authentication (hospitals, warehouses).

  - Quick device pairing.

## 10. Laptop Cameras & Microphones

- Built-in camera usually at top center of display.

- Laptops include microphones next to the camera.

- Can use external cameras for higher quality.

# ✅ Exam-Ready Summary (Quick Review)

- SO-DIMM = laptop RAM form factor.

- M.2 = small SSD interface used in modern laptops.

- Integrated battery requires full disassembly to replace.

- Cloning helps migrate data from HDD to SSD quickly.

- Antenna wires for Wi-Fi/Bluetooth run through display bezel.

- Biometrics (fingerprint/face) require OS + hardware support.

- NFC = very short-range wireless for authentication and payments.