Quiz2.1.17

1.What is the primary purpose of the Thunderbolt interface?

To enable high-speed data transfer and support multiple types of connections

Correct Answer:Correct

To connect wireless devices to a computer

### Explanation

The Thunderbolt interface is designed to provide high-speed data transfer, as well as support for various types of connections, including display, storage, and power through a single port. It combines PCI Express, DisplayPort, and power delivery in one interface.

2.A company is purchasing new laptops for their graphic artist division.

Which of the following display technologies provides better contrast and deeper blacks, resulting in better picture quality?

OLED

Correct Answer:Correct

### Explanation

Organic light-emitting diode (OLED) displays don't require a backlight. While this means they do not perform as well as LCD displays in lighted areas, they have a much better contrast ratio with deeper blacks.

Liquid crystal displays (LCDs) dominate the laptop display market but require a backlight and have a lower contrast ratio.

Digital light processing (DLP) is used in projectors.

While plasma display panels have a better contrast ratio than LCD displays, the technology is unsuitable for laptop displays because of their smaller size and larger power consumption.

3.Which of the following BEST describes a key feature of the DisplayPort interface that is used for connecting video monitors to computers?

DisplayPort can send both video and audio signals over the same cable.

Correct Answer:Correct

### Explanation

DisplayPort is an alternative to HDMI. Like HDMI, DisplayPort can send both video and audio signals over the same cable (if audio is supported by the video card and monitor).

DisplayPort uses a digital-only signal.

DisplayPort uses a different signal format than DVI or HDMI. However, DisplayPort supports sending DVI or HDMI signals over the same port using a simple adapter.

DisplayPort uses a different connector than HDMI. The connector is asymmetrical and has only one side beveled.

4.Which of the following is the maximum data transfer speed for USB 3.2 Gen 2 devices?

10 Gbps

Correct Answer:Correct

### Explanation

USB 3.2 Gen 2 specifies a maximum transmission speed of up to 10 Gbps. USB 2.0 devices have a maximum data transfer speed of 480 Mbps. USB 1.0 devices have a maximum data transfer speed of 1.5 Mbps or 12 Mbps.

5.You are an IT technician for your company. As part of your job, you must manage and support a wide variety of devices.

Which of the following is the MOST common interface for peripheral devices?

USB

Correct Answer:Correct

### Explanation

USB is the most common interface for peripheral devices, such as printers, keyboards, and mice.

FireWire is a common interface for digital video cameras and recorders, hard drives, and network adapters.

Serial interfaces are common for modems, older printers, and networking devices.

PATA interfaces are most common for internal HDDs and CD/DVD drives.

6.Which interface is primarily used for internal hard drives in modern desktop PC systems?

SATA

Correct Answer:Correct

### Explanation

SATA is primarily used for internal hard drives in modern desktop PC systems.

PATA (also called EIDE, IDE, and ATAPI) is a parallel ATA interface and was the most common interface used for hard disks and CD/DVD drives in the past. It is not used in modern PC systems.

USB and Firewire are interface standards for connecting various external devices, including external hard drives.

SCSI is commonly used for server storage. However, SCSI is rarely used for hard disks in modern desktop systems.

7.Susan works in the research and development department. She recently purchased a large high-speed external drive and has attached the drive to her computer using a USB cable. Her drive requires a minimum data transfer speed of 5 Gbps and needs to have access to 600 mA/15 volts of USB power to function properly.

Although the correct drivers are installed, when she plugs her drive into her workstation, the drive is not working. To troubleshoot the problem, she has connected her drive to her coworker's computer, where the drive functions properly. No additional cables are required for this drive.

Which of the following is the MOST likely reason that Susan's external hard drive is not working?

Susan has connected her drive to a USB 2.0 port, which does not have the required data transfer speed.

Correct Answer:Correct

### Explanation

Susan has connected her drive to a USB 2.0 port on her computer, which only supports up to 480 Mbps data transfer speeds. However, her external hard drive requires a minimum of 5 Gbps, which is provided by a 3.2 USB port.

Because the external hard drive requires 5 Gbps and 600 mA of USB power, the drive is most likely designed to be used with a 3.2 USB port.

A USB Type-C cable provides up to 20 volts of power, which meets the external hard drive requirements.

8.You are building a new computer and have purchased a motherboard that includes four built-in SATA connectors.

Which of the following MOST accurately lists the maximum number of SATA devices you can connect to the motherboard using the integrated ports?

Four

Correct Answer:Correct

### Explanation

The number of SATA devices that can be connected to a motherboard is directly determined by the number of built-in SATA connectors available, which in this case is four. Each SATA port supports a single device, such as a hard drive, solid-state drive, or optical drive. Since the motherboard in this scenario has four SATA connectors, it can accommodate up to four SATA devices simultaneously without the need for additional expansion cards or adapters. The option that eight, sixteen, or thirty-two devices can be connected is incorrect because they exceeds the number of available ports. To connect more than four devices, a SATA expansion card or port multiplier would be required, which is not mentioned in this scenario.

9.A user reports that he is unable to access data on an external hard drive. You investigate the problem and determine that the USB cable connection to the hard drive is damaged. You replace the cable.

Which of the following actions should you take next?

Test the solution.

Correct Answer:Correct

### Explanation

After implementing a solution, test it to verify full system functionality and ensure it has fully fixed the problem and hasn't caused any additional issues.

You document the problem and solution after everything has been fixed and verified.

You determine if escalation is needed and create an action plan before implementing the solution.

10.What is the maximum data transfer speed supported by USB 2.0?

480 Mbps

Correct Answer:Correct

### Explanation

USB 2.0, also known as Hi-Speed USB, supports a maximum data transfer speed of 480 Mbps (megabits per second). This was a significant improvement over the earlier USB 1.1 standard, which only supported speeds up to 12 Mbps. USB 2.0 is widely used for connecting peripherals such as keyboards, mice, printers, and external storage devices. While it is slower compared to newer standards like USB 3.0 and USB 3.1, USB 2.0 remains backward compatible with older USB devices and is still commonly found in many devices today.

11.A marketing manager needs to connect a laptop to a large wall-mounted LED TV in an auditorium to make a presentation that includes high-resolution video and audio.

Which of the following cable types would work BEST for connecting the laptop to the display?

HDMI

Correct Answer:Correct

### Explanation

HDMI is the default cable standard for newer electronic devices, such as Blu-ray players and LED TVs. HDMI can carry both digital video and audio signals. Most modern computers include an HDMI port.

DVI and VGA only support video signals.

S/PDIF cables and ports are used to send a digital audio signal to high-end audio devices. It does not support video signals.

12.Which connector on a computer is used to attach a mouse to a modern computer system?

USB

Correct Answer:Correct

### Explanation

Whether wired or wireless, a mouse is most commonly attached to a computer using a USB connector.

A DB-15 connector with two rows is commonly used for joysticks.

The DB-15 connector with three rows is used for connecting CRT monitors to the video card.

The RJ45 connector is used for Ethernet network connections.

13.Which of the following components is most likely to use a Molex power connector?

Case fan or older hard disk drive (HDD)

Correct Answer:Correct

### Explanation

Molex power connectors are commonly used to provide power case fans for older hard disk drive (HDD). Molex connectors are 4-pin power connectors traditionally found in older computer systems. They are often used to power devices such as case fans, optical drives, and older hard disk drives. They provide a reliable power connection but lack the more modern features found in newer power connectors.

A SATA SSD does not use a Molex connector. Instead, it relies on a SATA power connector, which is specifically designed for SATA-based drives and offers a more compact and streamlined design compared to Molex.

A monitor is powered externally through its own dedicated power cable and does not connect directly to the computer's internal power supply.

A USB external hard drive is powered through its USB connection or via an external power adapter.

14.Your company has just upgraded your laptop computer. After receiving your new computer, you plug your external USB 2.0 hard disk into your new USB 3.0 port.

Which of the following is the MOST accurate statement regarding how this device will function?

The device will work but only transfer at USB 2.0 speeds.

Correct Answer:Correct

### Explanation

USB 3.0 ports are backward compatible with all previous USB versions. However, these ports can only transfer as fast as the slowest version being used. This means that a USB 2.0 device connected to a USB 3.0 port will transfer data at USB 2.0 speeds.

Devices only require an external power source if they require more than 500 mA or 900 mA of power (for USB 2.0 and USB 3.0, respectively).

15.You are installing a new SATA hard drive in your Windows workstation.

Which of the following connectors should you use to connect your hard drive to the motherboard?

7-pin connector

Correct Answer:Correct

### Explanation

You use a 7-pin connector to connect a SATA hard drive to the motherboard.

IDE hard drives use a 40-pin connector.

SCSI and SPI-SCSI parallel interfaces use 50-, 68-, or 80-pin connectors.

Quiz2.2.18

1.How does the type of motherboard affect a computer system?

It allows any CPU model to be installed.

It affects the range of devices and adapter cards that can be installed.

Correct Answer:Correct

### Explanation

The type of motherboard impacts system speed and dictates which devices, CPU models, and adapter cards can be installed, affecting compatibility and expandability. Motherboards do not directly control RAM size for open applications, nor do they support any CPU model without compatibility considerations. Storage options are also influenced by the motherboard's specifications and connections.

2.You decided to upgrade your PC with a faster processor. To do this, you ordered a new motherboard over the internet that supports the processor that you want to use.

When the motherboard arrives, you discover that the motherboard uses the Micro-ATX form factor. Your current case is an ATX mid-tower with a standard ATX motherboard inside.

Which steps must you take for the Micro-ATX motherboard to work in the ATX case?

No additional steps are necessary. You can install the Micro-ATX motherboard in the ATX case.

Correct Answer:Correct

### Explanation

ATX mid-tower cases support all ATX form factors, including Micro-ATX. The main difference between ATX and Micro-ATX is the number of bus and memory slots on the motherboard.

You do not need to return the motherboard, and drilling holes in the motherboard or case can compromise or damage the integrity of both.

3. Why is mass storage necessary in a computer system?

To store only non-essential data temporarily

To preserve data and files when the computer is turned off

Correct Answer:Correct

### Explanation

Mass storage is necessary to retain data and files even when the computer is powered off. It does not affect CPU processing speed or control the system clock, and it is used for both essential and non-essential data rather than just temporary storage.

4.You are in the process of installing a motherboard in a system case.

Which of the following objects should you place between the motherboard and the system case?

Standoffs

Correct Answer:Correct

### Explanation

Standoffs go between the motherboard and the case. Standoffs prevent the motherboard circuits from touching the system case and grounding or shorting.

Heat spreaders go on memory modules to help cool them.

Fans are installed in the system case but not between the motherboard and the case.

Passive heat sinks are installed with chipsets and low-performance processors.

The support manual is a booklet that contains information about the motherboard.

20.Which of the following expansion buses uses a point-to-point dedicated connection and provides a serial full-duplex method of transmission?

PCIe

Correct Answer:Correct

### Explanation

PCIe was developed to replace PCI, PCI-X, and AGP. Instead of a shared bus, each PCIe slot links to a switch that prioritizes and routes data through a point-to-point dedicated connection and provides a serial full-duplex method of transmission.

The VESA local bus was a short-lived expansion bus that worked alongside the then-dominant ISA bus to provide a standardized high-speed conduit intended primarily to accelerate video (graphics) operations. VESA utilized a shared bus.

21.What distinguishes the M.2 interface from SATA regarding power requirements?

The M.2 interface supplies power over the bus.

Correct Answer:Correct

### Explanation

The M.2 interface provides power over the bus itself, so it does not require a separate power cable. Unlike SATA, which often needs a SATA or Molex power connector, M.2 simplifies installation by integrating power directly through the slot. It supports various devices, not just low-power ones.

22.What should be done when disassembling a computer to ensure correct reconnection of header connectors?

Document the position and orientation of connectors.

Correct Answer:Correct

### Explanation

Documenting the position and orientation of connectors using diagrams or photos is recommended to ensure correct reassembly, as labels can sometimes be small or difficult to interpret. Simply relying on connector color or assuming they fit one way can lead to improper connections.

23.The CEO of your small company has asked you to connect his laptop computer to the small conference room LED TV. The CEO will be showing a new promotional video that demonstrates the new company strategy through images and sound.

Which of the following cable types would work BEST for connecting his laptop to the display?

HDMI

Correct Answer:Correct

### Explanation

HDMI is the default cable standard for newer electronic devices, such as Blu-ray players and LED TVs. HDMI can carry both digital video and audio signals. Most modern computers include an HDMI port.

DVI and VGA only support video signals.

Composite cables support video but not audio.

24.Which connector is part of the system case and connects to header pins on the motherboard?

Power switch

Correct Answer:Correct

### Explanation

The system case typically includes wires that connect to the motherboard and provide the following functions:

- Power switch

- Reset switch

- Case speaker

The 24-pin ATX, SATA power, and 8-pin EPS12V connectors are provided by the power supply, not the system case.

25.Which of the following expansion slots are most commonly used for video cards in modern computer systems?

PCIe

Correct Answer:Correct

### Explanation

The PCIe (or PCI Express) expansion buses are most commonly used for video cards in modern computer systems.

PCI buses are most commonly used for devices such as sound cards, modems, network cards, and storage device controllers. While you can use PCI for video, better performance is obtained from PCI Express.

CNR is a legacy on-motherboard slot to support networking, wireless communication, sound, or modem functions.

AMR slots are legacy buses on some motherboards that are used by riser cards to support sound or modem functions.

26.You have just finished replacing the motherboard in your friend Ethan's computer with the same type of motherboard that the computer had originally. You tested the motherboard by powering it on and verifying that the operating system started and that the applications work.

Now Ethan is complaining that his computer no longer recognizes his external hard drive, which he had connected to the previous motherboard.

Which of the following is MOST likely causing the external hard drive not to function?

The USB front panel connector was not properly connected.

Correct Answer:Correct

### Explanation

The most likely cause is that the USB cable is not connected or is not connected properly.

If the motherboard is a replacement, and Ethan had the external hard drive connected previously, the new motherboard should also support the external hard drive.

It is unlikely that the hard drive suddenly failed in such a short time.

Since the operating system and applications are functioning properly, the power supply is functioning correctly as well.

27.Why is thermal paste applied when installing a CPU?

To improve heat transfer between the CPU and the heat sink

Correct Answer:Correct

### Explanation

Thermal paste is applied to improve heat transfer between the CPU and the heat sink, helping to manage the heat generated by the processor. It does not increase the electrical connection or secure the CPU physically, nor does it allow the CPU to operate at a higher voltage.

28.Why does a motherboard typically support only a limited range of CPU models?

Different CPUs require unique power delivery and socket designs.

Correct Answer:Correct

### Explanation

Modern CPUs are designed with specific electrical, physical, and technological requirements. These include the type of socket (e.g., LGA1200, AM4) and the power delivery systems that the motherboard provides. A motherboard must match these specifications to ensure proper communication and operation with the CPU. For example, an AMD Ryzen processor cannot be installed in an Intel motherboard because of incompatible sockets and architectures.

While the BIOS plays a critical role in recognizing and supporting a CPU, it can often be updated to support newer CPUs within the same socket and architecture.

Software restrictions are not typically the main factor in CPU compatibility. Hardware design (such as socket type, chipset, and power delivery) are the primary limits.

Performance level alone does not determine compatibility. Many motherboards are designed to support CPUs with varying performance levels within a given socket and architecture.

29.What is the primary function of a capture card?

To record or stream video input from various sources

Correct Answer:Correct

### Explanation

A capture card is primarily used to record or stream video input, such as gameplay or footage from HDMI sources, by saving it as a file or streaming it live. Generating an output for a monitor, increasing graphics processing power, and serving as the main video card in gaming computers are functions associated with graphics cards, not capture cards.

30.Which of the following objects is installed between the back side of the system case and the motherboard's ports?

I/O shield

Correct Answer:Correct

### Explanation

The I/O shield (also called a faceplate) is placed between the motherboard and the system case. The shield protects the case from dust and debris.

Standoffs are placed between the motherboard and the case and prevent the motherboard circuits from touching the system.

Fans are installed in the system case but not between the system case and the back I/O panel.

Heat spreaders are placed on memory modules to help cool them.

31.You are an IT professional tasked with setting up a conference room that includes both modern digital projectors and older analog video equipment.

You notice that the older equipment only has VGA ports, while the projectors have HDMI inputs.

Which solution would BEST address the connectivity issue using adapter cables?

Use a VGA to HDMI adapter cable to connect the older equipment to the projectors.

Correct Answer:Correct

### Explanation

Using a VGA to HDMI adapter cable to connect the older equipment to the projectors is appropriate because it involves using an adapter cable that converts the analog VGA signal from the older equipment to a digital HDMI signal compatible with the modern projectors.

HDMI to VGA adapters are less common and may not support the necessary signal conversion from digital to analog, which is more complex than the reverse.

Using a VGA to VGA cable to connect the older equipment directly to the projectors is not feasible because the projectors only have HDMI inputs, and a VGA to VGA cable would not be compatible with the HDMI ports.

Using an HDMI to HDMI cable to connect the projectors to the older equipment is not possible because, the older equipment does not have HDMI ports, so an HDMI to HDMI cable cannot be used for connectivity.

31.Which of the following statements accurately describes a key difference between DVI and VGA video cables?

VGA cables are designed for digital flat-panel displays, whereas DVI cables are used for cathode ray tube (CRT) monitors.

Both DVI and VGA cables are actively developed and commonly found on modern display devices.

DVI cables support both video and audio transmission, whereas VGA cables support only video.

Incorrect answer:Incorrect

DVI cables can support both analog and digital signals, while VGA cables support only analog signals.

Correct Answer:Correct

### Explanation

DVI supports both analog and digital outputs, whereas VGA is an analog-only interface. This accurately describes a key difference between the two cable types.

DVI and VGA support only video, not audio. Therefore, this statement is incorrect regarding DVI cables.

VGA was used for CRT monitors, which relied on analog signals, while DVI was designed to support both analog and digital outputs, making this statement incorrect.

DVI is no longer actively developed and is typically found on older display devices, and VGA is being phased out, making this statement incorrect.