Test 1:

1, Which of the following answers refer to common battery types that can be found in modern laptop devices? (Select 2 answers)

- Nickel-Iron (Ni-Fe)

- Lithium-ion (Li-ion) ( Your answer)

- Nickel–Metal Hydride (Ni-MH)

- Lithium-ion Polymer (LiPo) ( Your answer)

- Nickel-Cadmium (Ni-Cd)

You correctly answered this question.

Which laptop feature allows to overcome standard keyboard size restrictions?

- Touchpad

- Numeric keypad

- Fn key ( Your answer)

- Digitizer

You correctly answered this question.

What is the most common memory module form factor type used in laptops?

- ECC RAM

- DIMM

- SO-DIMM ( Your answer)

- C-RIMM

You correctly answered this question.

Which of the answers listed below refer(s) to internal data storage device type(s) used in laptop computers? (Select all that apply)

- Flash drives

- USB drives

- Magnetic disks ( Your answer)

- SSDs ( Your answer)

You correctly answered this question.

Which of the following answers refers to an HDD form factor for laptop computers?

- 1.8 inch

- 2.5 inch ( Your answer)

- 3.5 inch

- 5.25 inch

You correctly answered this question.

Which of the answers listed below refer to SSD form factors used in laptop computers? (Select 2 answers)

- 1.5 inch

- 5.25 inch

- 1.8 inch ( Your answer)

- 2.5 inch ( Your answer)

- 1.3 inch

You correctly answered this question.

Which of the following hardware components would be the most difficult to replace on a laptop computer?

- Optical drive

- Primary storage

- System board

- Integrated GPU ( Your answer)

- Secondary storage

You correctly answered this question.

Which laptop expansion card enables communication in 802.11 networks?

- WLAN ( Your answer)

- Riser card

- WPAN

- WWAN

You correctly answered this question.

Which of the answers listed below refers to a laptop's Bluetooth module?

- WWAN

- WLAN

- Riser card

- WPAN ( Your answer)

You correctly answered this question.

A type of laptop expansion card that enables communication over cellular network is referred to as:

- WPAN card

- WAN card

- WWAN card ( Your answer)

- WLAN card

You correctly answered this question.

Which of the following answers refer to the biometric authentication methods implemented in modern laptops? (Select 2 answers)

- Fingerprint scan ( Your answer)

- Near-field scanner features

- GPS reading

- Face recognition ( Your answer)

- Authenticator app

You correctly answered this question.

A technology implemented in modern laptops that enables data transfer and authentication features between the laptop and another portable device at a very close range is known as:

- NFC ( Your answer)

- Wi-Fi

- IoT

- WiMAX

You correctly answered this question.

Which of the answers listed below refers to a display technology most commonly used in modern laptops?

- LCD ( Your answer)

- OLED

- Plasma

- CRT

You correctly answered this question.

A type of backlight technology most commonly used in modern laptop devices is called:

- CCFL

- CRT

- OLED

- LED ( Your answer)

You correctly answered this question.

Which of the following answers refers to the location of a laptop's Wi-Fi antenna?

- On the WLAN module

- Near the top, inside display case ( Your answer)

- Inside laptop's case

- Typically attached via external expansion port

You correctly answered this question.

Which of the answers listed below describe the function of a laptop's inverter? (Select 2 answers)

- Converts AC power into DC power

- Used for supplying voltage to backlights in older types of LCD panels ( Your answer)

- Converts DC power into AC power ( Your answer)

- Used for supplying voltage to backlights in OLED displays

You correctly answered this question.

Docking stations: (Select 2 answers)

- Typically are designed to work with most laptop models (universal design)

- Usually offer additional ports and capabilities compared to port replicators ( Your answer)

- Typically are designed to work only with a specific laptop make and model (proprietary design) ( Your answer)

- Offer identical features as port replicators

- Typically offer less ports and capabilities than port replicators

You correctly answered this question.

What is the name of a laptop component that provides the function of a pointing device?

- Fn key

- Trackpad ( Your answer)

- Mouse

- KVM switch

You correctly answered this question.

Due to size restrictions and compact nature of laptops devices, this type of storage media becomes less commonly found on new laptops.

- Flash drives

- Optical drives ( Your answer)

- Solid-state drives

- Magnetic drives

You correctly answered this question.

Motherboards used in laptops and mobile devices can be characterized by their proprietary design, which means that installing a new motherboard in such devices most often requires replacement part designed for specific make and model of the given device.

- True ( Your answer)

- False

You correctly answered this question.